# 1. TEMPERATURE SENSOR READING USING LM35

### Aim:

To read the temperature using LM35 sensor and display the value on the serial monitor.

### Components Required:

- Arduino Uno

- LM35 Temperature Sensor

- Jumper wires

- USB cable

### Connections:

- LM35 VCC → 5V

- LM35 GND → GND

- LM35 OUT → A0

### Working Principle:

LM35 gives output voltage proportional to temperature.
Every 1°C = 10mV.
Arduino reads analog value and converts it to temperature.

### Code:

int sensor = A0;

void setup() {

Serial.begin(9600);

}

void loop() {

int value = analogRead(sensor);

float temperature = value * 0.488;

Serial.println(temperature);

delay(1000);

}

# 2. TEMPERATURE IN FAHRENHEIT WITH MIN & MAX VALUE

### Aim:

To display temperature in Fahrenheit and also track minimum and maximum temperature.

### Connections:

Same as Program 1.

### Working:

Temperature is converted from Celsius to Fahrenheit using formula:
F = (C × 9/5) + 32

### Code:

int sensor = A0;

float minTemp = 1000;

float maxTemp = 0;

void setup() {

Serial.begin(9600);

}

void loop() {

float c = analogRead(sensor) * 0.488;

float f = (c * 9 / 5) + 32;

if (c < minTemp) minTemp = c;

if (c > maxTemp) maxTemp = c;

Serial.print("Temp(F): ");

Serial.print(f);

Serial.print("  Min: ");

Serial.print(minTemp);

Serial.print("  Max: ");

Serial.println(maxTemp);

delay(1000);

}

# 3. TEMPERATURE GRAPH USING SERIAL PLOTTER

### Aim:

To plot temperature values graphically using Arduino Serial Plotter.

### Connections:

Same as LM35 connection.

### Working:

Serial Plotter plots values continuously when only numbers are printed.

### Code:

int sensor = A0;

void setup() {

Serial.begin(9600);

}

void loop() {

float temp = analogRead(sensor) * 0.488;

Serial.println(temp);

delay(500);

}

# 4. LED BLINKING APPLICATION

### Aim:

To blink an LED at regular interval.

### Components:

- LED

- 220Ω Resistor

- Arduino

### Connections:

- LED positive → Pin 13 (via resistor)

- LED negative → GND

### Code:

void setup() {

pinMode(13, OUTPUT);

}

void loop() {

digitalWrite(13, HIGH);

delay(500);

digitalWrite(13, LOW);

delay(500);

# }

# 5. TEMPERATURE THRESHOLD ALERT USING LED

### Aim:

To turn ON LED when temperature crosses a threshold.

### Connections:

- LM35 → A0

- LED → Pin 8

### Working:

If temperature > 30°C, LED turns ON.

### Code:

### int sensor = A0;

### int led = 8;

### void setup() {

### pinMode(led, OUTPUT);

### }

### void loop() {

### float temp = analogRead(sensor) * 0.488;

### if (temp > 30)

### digitalWrite(led, HIGH);

### else

### digitalWrite(led, LOW);

### delay(500);

### }

# 6. OBSTACLE DETECTION USING ULTRASONIC SENSOR

### Aim:

To detect obstacle using ultrasonic sensor and alert using LED.

### Connections:

- Trig → 9

- Echo → 10

- LED → 8

### Code:

int trig = 9;

int echo = 10;

int led = 8;

void setup() {

pinMode(trig, OUTPUT);

pinMode(echo, INPUT);

pinMode(led, OUTPUT);

}

void loop() {

digitalWrite(trig, LOW);

delayMicroseconds(2);

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

long duration = pulseIn(echo, HIGH);

long distance = duration * 0.034 / 2;

if (distance < 20)

digitalWrite(led, HIGH);

else

digitalWrite(led, LOW);

delay(200);

}

# 7. DOOR OPENER USING SERVO AND ULTRASONIC SENSOR

### Aim:

To automatically open door when object comes near.

### Connections:

- Servo → Pin 6

- Ultrasonic → Trig 9, Echo 10

### Code:

#include <Servo.h>

Servo door;

int trig = 9, echo = 10;

void setup() {

door.attach(6);

pinMode(trig, OUTPUT);

pinMode(echo, INPUT);

}

void loop() {

digitalWrite(trig, LOW);

delayMicroseconds(2);

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

long d = pulseIn(echo, HIGH) * 0.034 / 2;

if (d < 15)

door.write(90);

else

door.write(0);

}

# 8. RGB LED CONTROL USING POTENTIOMETERS

### Aim:

To control RGB LED color using 3 potentiometers.

### Connections:

- Pot1 → A0 (Red)

- Pot2 → A1 (Green)

- Pot3 → A2 (Blue)

- RGB pins → 9, 10, 11

### Code:

### void setup() {}

### void loop() {

### analogWrite(9, map(analogRead(A0), 0, 1023, 0, 255));

### analogWrite(10, map(analogRead(A1), 0, 1023, 0, 255));

### analogWrite(11, map(analogRead(A2), 0, 1023, 0, 255));

### }

# 9. IoT TEMPERATURE & HUMIDITY MONITORING (DHT11)

### Aim:

To monitor temperature and humidity using DHT11 sensor.

### Connections:

- DHT11 Data → Pin 2

- VCC → 5V

- GND → GND

### Code:

### #include <DHT.h>

### #define DHTPIN 2

### #define DHTTYPE DHT11

### DHT dht(DHTPIN, DHTTYPE);

### void setup() {

### Serial.begin(9600);

### dht.begin();

### }

### void loop() {

### float temp = dht.readTemperature();

### float hum = dht.readHumidity();

### Serial.print("Temperature: ");

### Serial.print(temp);

### Serial.print(" °C  ");

### Serial.print("Humidity: ");

### Serial.println(hum);

### delay(2000);

### }